|  | SCAR 81 VIRTUAL FIGHTER SQUADRON TACTICS, TECHNIQUES, AND PROCEDURES FOR STRIKE COORDINATION AND RECONNAISSANCE for DCS WORLD OPERATIONS FEBRUARY 2021 DISTRIBUTION STATEMENT: Distribution authorized to 81 VFS and JTF-1 members only. Any release outside of those organizations requires prior approval. |
| --- | --- |

CONTENTS

1	DEFINITIONS	4

1.1	Introduction	4

1.2	Strike Coordination and Reconnaissance	4

1.3	SCAR Aircrew Responsibilities	4

1.4	SCAR Aircrew Functions	5

2	BREVITY	6

2.1	Introduction	6

2.2	General	6

3	PROCEDURES	7

3.1	Introduction	7

3.2	Mission Briefing	7

3.3	Search for Targets	7

3.4	Asset Deconfliction	8

3.5	Strike asset Check-in	13

3.6	SCAR Execution	14

3.7	Communication Flow Examples	19

4	SCAR MISSION PLANNING GUIDE	22

4.1	Overview	22

4.2	Friendly Situation	22

4.3	Communications	22

4.4	Intelligence Overview	22

4.5	Weaponeering	22

4.6	Reconnaissance	22

4.7	Target Area Tactics	22

4.8	Navigation Plan	23

4.9	Go/No-Go Criteria	23

5	SCAR MISSION BRIEFING GUIDE	24

5.1	Mission Objectives	24

5.2	Threat Counter-Tactics	24

5.3	Air tasking Order/Schedule Overview	24

5.4	Working Area/Map/Imagery Study	24

5.5	Ingress Routing	24

5.6	SCAR Plan	25

5.7	SCAR Coordination	25

5.8	SCAR Tasking	26

5.9	Battlefield Handover	26

5.10	Egress	27

5.11	Contingencies	27

6	SCAR DECONFLICTION CARD	28

7	SCAR ATTACK BRIEF (DLER Format)	29

# DEFINITIONS

## Introduction

This Joint Tactics, Techniques, and Procedures publication provides fundamental employment procedures and techniques for 81st Virtual Fighter Squadron pilots when operating as SCAR aircrew.  This publication serves as a reference for planners, and a checklist for operators to execute SCAR aircrew missions.

SCAR missions are flown in a specific geographic area and are elements of command and control (C2) used to match weapons effects with targets per the mission prioritized target list through real-time employment of air interdiction (AI) assets. SCAR missions are designed to effectively and efficiently destroy targets and conduct associated battle damage assessments (BDAs).  This document gives an insight on useful Tactics, Techniques, and Procedures that you can apply effectively in DCS.

## Strike Coordination and Reconnaissance

SCAR is defined as “A mission flown for the purpose of detecting targets and coordinating or performing interdiction or reconnaissance on those targets.”

SCAR missions are flown in a specific geographic area and are elements of command and control (C2) used to match weapons effects with targets per the 81 VFS prioritized target list through accurate real-time allocation of interdiction assets. SCAR missions are designed to effectively and efficiently destroy targets, conduct associated battle damage assessments (BDAs), and pass on BDA to command and control.  Using correct terminology allows threat calls to be short, crisp, and timely. See operational brevity words contained in 476TTP 3-1.Brevity.

## SCAR Aircrew Responsibilities

SCAR aircrew are responsible for accomplishing three key duties and must be capable of executing these duties’s in day, night and adverse weather conditions.  These duties include:

### Deconflicting all assets within the assigned airspace/killbox;

- Deconfliction of supporting fixed-wing or rotary-wing asset.

- Deconfliction of ordnance delivered from those assets.

### Coordinate location, identification, engagement and assessment of targets to achieve the mission objectives;

- Ensure ROE/SPINS and mission commander’s requirements on the target(s) have been satisfied.

- Find, fix, track, target, engage, and assess targets using supporting assets and own-ship capabilities.

- Prioritize targets IAW the commander’s direction.

- Assess threats through available means and mitigate them.

- Conduct reconnaissance and provide situation updates to supporting assets.

- Pass target information to supporting assets using either the preferred description, location, elevation, and remarks and restrictions (DLER) attack brief format.

- Coordinate, sequence, and direct aircraft.

- Match desired weapons effects with targets.

- Integrate supporting asset capabilities, as required.

- Coordinate BDA of targets and reattack, as required.

- Mitigate redundant strikes against previously attacked targets.

- Coordinate for additional assets, as required.

### Provide a battlefield handover briefing to the follow-on SCAR aircrew, and provide reports in the 81st VFS operational forum thread.

## SCAR Aircrew Functions

It is the expected that SCAR aircrews execute certain functions, those functions are:

- Provides target marks (e.g., infrared (IR) pointer, laser, or rockets).

- Serves as an intelligence, surveillance, and reconnaissance (ISR) collection platform.

- Performs AI, as required.

- Monitors area of interest to provide judicious warning of enemy location and activity.

- Provides terminal guidance for laser-guided weapons.

- Assists with BDA.

# BREVITY

## Introduction

The following tables show some of the brevity that is used in SCAR operations. For a list of all brevity in use, consult 476TTP 3-1.Brevity, available on our website.

## General

| BREVITY TERM | DEFINITION |
| --- | --- |
| INVESTIGATE | Verify specified element(s) of rules of engagement (ROE), positive identification (PID), collateral damage estimation (CDE), and/or coordination of forces on the referenced target or track. |
| TARGET | ROE, PID, coordination of forces, and commander’s guidance requirements on the referenced target/track have been satisfied. Target/track correlation and CDE must be accomplished prior to employing ordnance. |
| SMACK | Clearance to employ ordnance/fires on surface targets’ coordinates. ROE, PID, CDE, coordination of forces, and commander’s guidance requirements on the referenced target/track have been satisfied. Not to be utilized in a close air support engagement. |

# PROCEDURES

## Introduction

Prior to acting as SCAR aircrew within the 81st pilots must have a thorough understanding of the prioritized target list, enemy threats, mission working area/kill box, and the capabilities and limitations of supporting assets.

SCAR aircrew within the 81st is made up of one 2-ship element.  The lead being the primary SCAR aircrew and their wingman providing mutual support.

## Mission Briefing

Mission briefings must convey the SCAR aircrews plan to all air assets.  Begin the briefing by providing an overview of the objectives, package composition/munitions, and administrative procedures (routing, holding, etc.).  Refer to Appendix A “SCAR Mission Briefing Guide” for specifics.

The briefing should cover the overall plan with the tactical situation, targets and known threats.  SCAR aircrews must be prepared to provide this information when in theatre to aircraft that may not have been able to participate in the initial briefing.  Upon first contact SCAR aircrew information flow should include:

- AO entry/check-in

- Situational update/SITREP

- Deconfliction

- Asset tasking

- BDA/INFLTREP

- Battlefield handover

## Search for Targets

- Use big-to-small focused patterns while using all available sensors.

- Analyze in wide field of view, confirm in narrow.

- Assess the benefits of revisit plans versus overhead loiter.

- Use multiple sensor sources and angles to build a better operational picture

### Search Considerations

- Altitude. The search altitude will vary based on threat, sensor selection, and time of day. Search altitudes are a trade-off between threat reduction and sensor performance. Don’t completely rely on sensors, visual searches should be used as well when locating the enemy due to the large field of view that can be covered, the ability to perceive movement when searching along lines of communications, and the ability to capture smoke or dust trails.

- Sensor-Aided Search. Sensor search techniques will vary based on the environment within the assigned geographic area. The SCAR aircrew should plan to use all available assets to aid in searching the SCAR mission working area.

- Points Of Interests (POI). An additional technique is to split up several POIs between flights and supporting assets or within the SCAR element/section. For example, with 10 POIs, the SCAR aircrew would search POIs 1–5, and the additional flight would search POIs 6–10. The SCAR aircrew could also assign POI geographically. For example, the SCAR aircrew could search a valley while another flight searches a plateau. Either way, a systematic method will reduce search time and increase the area covered by the SCAR aircrew and supporting assets.

- Communication. While the SCAR aircrew is on station, other assets may find potential targets and pass information to the SCAR aircrew. This information may be passed via voice or data transmission in the form of an attack brief, as described in below. At a minimum, information should include a target location and a brief description. Also, the SCAR aircrew must know the proper communication chain to request additional assets, if required.

## Asset Deconfliction

SCAR aircraft must ensure all assets operating within their area of responsibility are safely deconflicted at all times.   The holding/deconfliction plan must take into consideration enemy capabilities, target orientation, weather, known threats, and strike asset capabilities.  If possible avoid leaving strike aircraft idle and use their sensor/weapon capabilities during time on station whether in the hold or during task execution.

### Deconfliction Assumptions

#### Fighters

- Plan a minimum 2,000 foot (ft) deconfliction block for 2-Ship and 4,000 ft for 4-Ship.

- Typically altitude, is 5,000–25,000 ft (Altitude varies based on threats, average terrain, and munitions)

  - This altitude block is a good tradeoff between on-station time and sensor capability.

- 15-by-15 nm holding area is sufficient airspace for most fighters.

#### Helicopters

The SCAR aircrew should consider the fragmentation envelope and bomb fall line of concurrent strike aircraft in relation to the helicopter holding position to prevent fratricide.  Helicopters normally operate below 10,000 ft MSL and at a groundspeed of 70–150 kts.

- High threat altitude profile may be 50–200 feet above ground level (AGL)

- Medium/low threat altitude profile may be 1,500–2,500 feet AGL.

- Helicopter holding areas are established at distances from the target area that provide adequate threat stand-off.

Figure X-X, Altitude Deconfliction V- Diagram

Deconfliction can be accomplished by:

- Lateral. Lateral deconfliction separates assets in geographical space. References that may be used to separate assets include terrain features, kill box, or coordinates (MGRS for example).

- Altitude. Altitude deconfliction separates assets in vertical space. If operating in a limited area, it will be necessary to stack elements in the same airspace and deconflict them by altitude. The SCAR aircrew should stack all aircraft to optimize on station time, sensor performance, expedite deconfliction of attacks, and mitigate the threat. Refer to figure 15 for an example of altitude deconfliction.

- SCAR Stacked Low is a technique that increases the probability of achieving target positive identification and enables immediate attack.

- SCAR Stacked High is a technique that provides the SCAR aircrew the autonomy to move within the mission area/kill box with minimal deconfliction.  SCAR aircrew must ensure deconfliction should they conduct strikes through lower asset assigned lower altitudes.

- Time. Time deconfliction may be necessary if lateral and altitude deconfliction options are limited.

- Combination of Lateral and Altitude. Deconfliction methods may be combined to increase the efficiency and effectiveness of the mission.

### Holding Patterns

When possible, aircraft should be given enough airspace to hold in an area of relatively low AAA activity that provides a good position to observe the target area. SCAR aircraft should not restrict attack aircraft to specific observation or holding patterns but should specify the observation or holding area that will best accomplish the mission.

Considerations for observation or holding area and altitude selection include: weather conditions, terrain and threat locations and activity, and other attack aircraft either on station or inbound.

Typical holding patterns include the following:

- Racetrack: An oval holding pattern with straight legs of at least 10 miles in length and hard, 180 degree turns on each end.

- Figure Eight: The same as the racetrack pattern except the turns at each end of the pattern are made toward the target area and are 230 degrees of turn instead of 180 degrees.

- Wheel: Circle around the designated target. Ideal for non-linear battlefields with “pockets” of enemy activity.

## Strike asset Check-in

At the CP, the flight contacts the SCAR aircrew for check-in (do not start your check-in on first communications, but wait for the “ready for check-in” call.) The check-in of strike aircraft is contained in the acronym MNPOPA:

- Mission number

- Number and type of aircraft

- Position

- Ordnance

- Playtime

- Abort code

For LGBs the laser code has to be included in the check-in brief. The type of sensors and pilot capabilities are given in the remarks section when available (SCAR qualified, sensors, SADL, Link-16, ...)

(Aircraft transmits to SCAR Aircrew)

Aircraft: _______________________, this is: _____________________
         (SCAR aircraft call sign)          (Strike Aircraft

Call sign)

NOTE: Authentication happens here

1. Mission number: ______________________________________________

2. Number and type of aircraft: _________________________________

3. Position and altitude: _______________________________________

4. Ordnance: ____________________________________________________

5. Time on station: _____________________________________________

6. Abort code: __________________________________________________

Remarks: ________________________________________________________

After the check-in, the SCAR aircrew will provide a situation update if required. This update includes target/enemy situation, threat activity and friendlies.  Table X-X shows the items that should be covered during the situation update.

| SITUATION UPDATE LINE | SITUATION UPDATE | BATTLEFIELD HANDOVER (In addition to info passed in Situation Update) |
| --- | --- | --- |
| Threat Activity | Threats observed: who, what, where, when. |  |
| Enemy Situation | Provide the current enemy disposition. Provide the size, activity, & location Give Specific targets and locations, if available. | Include target priorities Include target location grids |
| Friendly Situation | SCAR, striker assets on station (time on station & ordnance) |  |
| Coordination | Brief/determine who has engagement authority |  |
| Hazards | Weather, terrain, or obstructions |  |
| Remarks/Restrictions | Updates on current aircraft location, altitude, & tasking(s) | Sensor management plan |
| Note: The SCAR aircrew will immediately pass routing/deconfliction for inbound striker assets prior to situation update. Use either the targets, threats, friendlies, clearance, ordnance, restrictions format. |  |  |

Table X-X Situation Update & Battlefield Handover

## SCAR Execution

### Target Detection

In general the following are tools the SCAR aircrew can use to maximize target acquisition.

- Use available intelligence for initial search

- Use available on-board sensors such as targeting pods, air-to-ground radar, NVGs, etc.

- Use supporting assets/strikers effectively to maximize search area and target acquisition.

Focus on areas where enemy assets can be concealed or fortify their positions.  Assign supporting assets search areas based on points of interest, quadrants, geographical features, or coordinates.  As targets are found they need to be identified, strategized, annotated, and categorized based on the priority target list.  As a SCAR aircrew you are required to balance target engagement with priorities, available assets, their ordnance, and their time on station.  Once this is completed take into consideration the targets location and acquiring the accuracy required to coordinate the strike.

### Tracking & Engagement Decisions

Use sensors to continue to monitor and track the target to maintain positive identification.  As a SCAR aircrew you are required to evaluate the target’s window of vulnerability during the tracking process. Mobile targets may require immediate engagement and not require complete destruction of all targets to get the desired outcome.  If the decision is made to delay the strike and continue tracking the target it should only be for the following reasons:

- Threats in the area

- Gather more or exploit current information

- Observe enemy activity

- Awaiting clearance or guidance from Mission Commander

- To avoided collateral damage

- Unsuitable weapon-to-target match

- Engage higher priority targets

SCAR aircrew have a duty to pair strike assets with targets they can achieve the highest probability of success within the anticipated timeframe. This should be based on an understanding of specific asset/weapon capabilities and limitations in addition to the desired weapons effects for the target.  Other considerations include:

- Asset situational awareness and location

- Asset time on station

- The target’s window of vulnerability

- Collateral damage

If threats pop up it is the SCAR aircrew’s responsibility to guide strike assets away from the threat(s) then:

- Determine the extent of the threat’s effect on the area/killbox.

- Determine if the threat can be isolated by altitude or lateral separation.

- Identify SEAD capabilities and coordinate SEAD strike

- Notify on station and/or incoming assets.

### Attacks

SCAR aircrew will pass on attack brief briefings to include data link, GEOREF, or coordinates.  Description, Location, Elevation, and Remarks/restrictions is the preferred attack briefing format.

- INVESTIGATE is used to optimize sensors to gather target information such as location, PID, composition, and disposition. It is also used to gain situational awareness of the mission working area.

- TARGET is used to create effects on known or suspected targets when the striker asset can satisfy all requirements prior to target engagement.

- SMACK is used to create effects on known targets I when the strike aircraft cannot satisfy all requirements prior to target engagement.

| INFORMATION | DESCRIPTION |
| --- | --- |
| Description | Include type of targets, size, disposition, and other pertinent data. |
| Location | Options include Lat/Long, MGRS, talk-ons, SADL messaging, GEOREF |
| Elevation | Feet mean sea level |
| Remarks/Restrictions | Deconfliction measures, ordnance, final attack heading, tone on target window, restrictions. |

Table X-X, DLER Attack Briefing Format

Air attack brief example.

SCAR:		Panther 5-1, Cylon 1-1, advise when ready for TARGET tasking.

Flight lead:		Panther 5-1, ready.

SCAR:		Panther 5-1, TARGET, 1 BMP, 36S YF 497 321, elevation 129 ft, make attack north to south, remain west of town.

Flight lead:	Panther 5-1.

#### Coordinated Attacks

Assets must be synchronized, not only with other assets, but with any supporting arms that we are planning to utilize. In a permissive threat environment, timing may not be as critical, and (if we have properly coordinated the airspace over a target) we can have the aircraft attack in sequence or at random, either from an overhead pattern, or from a holding pattern offset from the target area. In a non-permissive threat environment, we will be required to synchronize the assets with SEAD and with each other. Because of the SEAD requirement, we would prefer to have the players attacking simultaneously or sequentially, with minimum white space between attacks. Random attacks are impractical in this situation and would probably leave attacking aircraft without suppression.

Coordinated attacks that we might utilize include:

- Multiple aircraft using timing splits over the target.

- Multiple elements using a combination of timing and heading splits.

- Multiple flights using dissimilar aircraft (fixed wing attack/bomber and rotary wing aircraft) in coordinated attacks.

Coordinating flights for attacking the same target/target area can add firepower to the attack and help to split target defenses.

##### Attack Types

There are two types of attacks that can be utilized in an objective area in regards to synchronization of fire support assets.

- Combined Attacks. In this type of attack, the avenue to the target and/or the area over the target is shared airspace. During this attack, aircraft will be flying over the same area. Though lateral deconfliction may not be present, aircraft still may be deconflicted by altitude or timing.

- Secretor Attacks. In this attack method, the avenue to the target, and/or the area over the target or targets is sectored between attacking aircraft. Visible or invisible demarcation lines (distinct roads, rivers, grid lines, etc.) are acknowledged by all players and utilized for lateral deconfliction. Additional deconfliction via altitude and timing may also be utilized.

| TYPE OF ATTACK |  | SIMULTANEOUS | SEQUENTIAL | RANDOM |
| --- | --- | --- | --- | --- |
| Combined | Same avenue of attack | Visual or timing Simultaneous time on target or time to target | Visual or timing (visual spacing or time separation) | Not normally used |
| Sector | Acknowledged sector | Visual or timing Simultaneous time on target or time to target | Visual or timing (visual spacing or time separation) | Free flow (must ensure strafe or bomb frag deconfliction) |

Table X-X, Coordinated Attacks

##### Attack Timing

There are three types of timing that can be utilized to synchronize attacks on targets in an objective. They can be combined with the previously listed attack types.

- Simultaneous.  As the name implies, all elements attack at the same time. This type of timing has the advantage of massing fires, maximizing shock effect, complicating the enemy AD targeting scheme and being unpredictable. Unfortunately, it may also complicate target array sorting and direct fire planning, and simultaneous weapons impacts may interfere with one another.

- Secretor Attacks.  Timing for this attack is in a predetermined sequence for all players. This is advantageous to friendlies because the initial attack in the sequence marks the target area for subsequent attackers, it puts continuous pressure on the target over a period of time, produces less interference from weapons of other shooters, insures that individual targets are not “double-targeted,” and is the preferred method for multiple fixed wing flights. This method of timing does potentially allow enemy AD assets to target individual aircraft or sections as they attack, it also takes longer to accomplish, thereby reducing the shock effect and perhaps providing opportunities for enemy exploitation.

- Random. In a random attack, all elements attack at will. Some benefits of this type of attack timing are its ease of use for pilots, requires less command and control, and is unpredictable for the enemy. It can be disadvantageous because it requires more extensive aircraft and weapons deconfliction, may let pressure off of the enemy and can otherwise complicate the fire support plan.

###### Mobile Target Considerations

Mobile targets may require the SCAR aircrew to adjust the airspace assigned for the coordinated attack. SCAR aircrew needs to anticipate this possibility and be prepared to coordinate additional airspace or update the deconfliction plan. If the the attack method, timing, or assets conducting a coordinated attack during an engagement need to be adjusted, to ensure safety of all aircraft, the SCAR aircrew may need to cease the current coordinated attack prior to making the changes.

###### Attacking Aircraft Egress

When the attack is completed, attack participants will egress along a route that minimizes threat exposure. During egress, the SCAR aircrew can assist aircraft egressing by pointing out threats and provide updated egress routing instructions, as necessary.

### Battle damage Assessment (BDA)

The last step in the sequence of events is to assess the effects of the attack.  BDA is used to update the enemy order of battle. Accurate BDA is critical for determining if a target should be reattacked.  In a high-threat environment, BDA may be difficult to judge.

#### Strike aircraft BDA

Strike aircraft are responsible for conducting their BDA to determine if reattacks are necessary.  The strike aircraft will report their BDA to the SCAR aircrew when completed using the below format.

(Aircraft transmits to SCAR Aircrew)

Aircraft: _______________________, this is: _____________________
         (SCAR aircraft call sign)            (Strike Aircraft

Call sign)

NOTE: Authentication happens here

1. Strike Callsign: ______________________________________________

2. Mission Number: ______________________________________________

3. Position and altitude: _______________________________________

4. Time-on-Target: ______________________________________________

5. Results: _____________________________________________________

Remarks: ________________________________________________________

(Target area weather, signification observations, essential info)

#### SCAR Aircrew

SCAR aircrew determine if additional attacks are required based on strike aircraft BDA.  If additional attacks are required the SCAR aircrew will task on station strike aircraft or request additional assets.

### Battlefield Handover

Perhaps the most important thing we can do in the turnover process is to plan our fuel accordingly. We should have enough gas and time on station remaining to be able to perform an effective SCAR aircrew to SCAR aircrew turnover. We should strive NOT to abruptly declare we are RTB due to a bingo fuel state, and have to perform a hasty turnover as we egress the target area. Our goal should be to enhance the overall situational awareness of the new SCAR aircrew. He should have a good understanding of the overall situation, the known or observed threats, the ground scheme of maneuver, the comms plan, the ground commander’s objectives, and game plans which have been working for successful attacks.  Use the table X-X for minimum Handover items.

The off-going SCAR aircrew should also provide a detailed list of targets previously attacked, target numbers, and targets remaining to be attacked. Additionally, inform him of what GeoRef or visual talk-on aids were useful. All assets on station should be described, including remaining ordnance and TOS, as well as any special equipment or capabilities / degrades.

Once our turnover is complete, as the off-going SCAR aircrew, we should deconflict ourselves from other airborne assets and inform everyone how we are egressing from the target area.

## Communication Flow Examples

### Fighter Check-in

Example:

Flight lead:	Cylon 1-1, Panther 5-1.

SCAR:	Panther 5-1, Cylon 1-1, go ahead.

Flight lead:	Ready for check-in.

SCAR:	Proceed with check-in.

Flight lead:	Mission number one three three seven. Two A ten charlies. Eight miles east of contact point, at sixteen-thousand. Each has four by Mk-82, instantaneous fuzing, one by AGM-65 golf, one by GBU-12, and full gun. Laser code dash one: one five five one, dash two: one five five two. Nine zero minutes playtime. Abort code for dash one: november lima, dash two: foxtrot uniform. Targeting pod and SADL equipped.

SCAR:	Copy all. Proceed and hold at alpha mike two four six eight. Right hand racetrack, heading two five five, ten mile legs, angels sixteen. Report established.

Flight lead:	Alpha mike two four six eight. Hold a right hand racetrack, heading two five five, ten mile legs, angels sixteen, and report established. Panther 5-1.

Flight lead:	Cylon 1-1, Panther 5-1, established.

### SITREP and Game Plan

Example:

SCAR:	Panther 5-1, sitrep as follows, I have friendly armor on the south bank of a large east/west river maintaining overwatch on a grey arched-steel road bridge. The bridge is located about one klick east of a sharp bend in the river to the south. The ground commander’s intend is to protect that bridge and repel any opposing forces. I’m tally on a mechanized infantry company parked on the north-to-south hard ball about five klicks north of the bridge. I count ten BMPs and numerous dismounts. I need multiple gun runs on those vehicles and personnel.

Flight lead:	Panther 5-1 copies.

### INVESTIGATE Taskings

#### INVESTIGATE tasking; engagement NOT authorized, recon only

Example:

SCAR:		Panther 5-1, Cylon 1-1, advise when ready for INVESTIGATE tasking.

Flight lead:		Panther 5-1, ready.

SCAR:		Panther 5-1, INVESTIGATE 36S YF 497 321, suspected enemy armored column moving north to south, report when complete.

Flight lead:	Panther 5-1.

#### Engagement authority not authorized, recon only

Example:

SCAR:		Panther 5-1, Cylon 1-1, advise when ready for INVESTIGATE tasking.

Flight lead:		Panther 5-1, ready.

SCAR:		Panther 5-1, INVESTIGATE 36S YF 497 321, suspected enemy armored column moving north to south.  If BMPs found, you are approved to TARGET. Make attack north to south.

Flight lead:	Panther 5-1.

### TARGET Taskings

#### TARGET single strike

Example:

SCAR:		Panther 5-1, Cylon 1-1, advise when ready for TARGET tasking.

Flight lead:		Panther 5-1, ready.

SCAR:		Panther 5-1, TARGET, 10 BMPs, 36S YF 497 321, elevation 129 ft, make attack north to south, remain west of town.

Flight lead:	Panther 5-1.

#### TARGET multiple strike assets; sectored sequential coordinated attack:

Example:

SCAR:		Panther 5-1 and Ford 2-1, Cylon 1-1 advise when ready for TARGET tasking.

Flight lead:		Panther 5-1, ready.

Flight lead:		Ford 2-1, ready.

SCAR:		Target set is four BMPs, two T-55s, one ZSU 23 traveling west to east on hardball, 3 miles east of the town. Coordinated attack, sectored-sequential-timing, Ford followed by Panther. Ford 2-1, make attack east to west on ZSU 23-4, egress west. Panther 5-1, make all attacks south to north on remaining targets, remain 5 nm south of targets until Ford 2-1’s attacks are complete, TOT (time on target) 50.

Flight lead:	Panther 5-1.

Flight lead:		Ford 2-1, ready.

# SCAR MISSION PLANNING GUIDE

## Overview

- Review rules of engagement (ROE), special instructions (SPINS), the collateral damage limitations for the area of operations (AO) or target area, and air operations directive (AOD).

- Review the airspace control order (ACO) for kill boxes, airspace coordinating measures (ACMs), and restricted operating zones (ROZs).

## Friendly Situation

- Review the air tasking order (ATO) for mission-pertinent data to include determining:

  - The SCAR mission working area.

  - The SCAR, strikers, and supporting assets.

  - Reconnaissance assets.

  - Offensive counterair (OCA) and defensive counterair (DCA) assets.

  - Suppression of enemy air defense (SEAD) assets.

  - Refueling assets.

- Determine command and control communication requirements.

## Communications

- Check in and out and inflight report procedures (i.e., agency and frequency).

- SCAR working frequencies.

- Data link considerations.

## Intelligence Overview

- Review suspected enemy locations and identify points of interest (POIs).

- Assess the enemy’s air and missile threats.

## Weaponeering

- Identify potential targets and select appropriate weapons.

- Develop a weapon load plan.

- Plan valid deliveries for ordnance.

## Reconnaissance

Develop a search profile considering the following.

- Target priorities.

- Expected target locations.

- The threat.

- Predicted sensor detection ranges.

## Target Area Tactics

- Study target area imagery.

- Assess the impact of the following on target area tactics.

  - Sensor capabilities and limitations.

  - Threat.

  - Weather.

  - Terrain.

- Develop a holding/deconfliction plan based on ATO-assigned airspace and assets.

- Consider optimum attack axes based on expected target orientations and environmental influences on target acquisition.

- Prepare attack brief templates for all striker assets.

- Develop a fires integration plan.

## Navigation Plan

- Waypoints.

  - Ingress and egress routing.

  - Sensor POIs, named area of interest (NAI), and target area of interest (TAI).

  - BULLSEYE.

  - Initial points (IPs).

  - Holding Points.

## Go/No-Go Criteria

- Aircraft system requirements.

- Weather.

- Supporting assets.

# SCAR MISSION BRIEFING GUIDE

## Mission Objectives

- Commander’s intent/desired effects in the SCAR mission working area.

- Responsibilities.

  - Navigation.

  - Fires and airspace Deconfliction.

  - Check-in briefs.

  - Stack management.

- Systems and sensors.

  - Visual lookout.

  - Targeting pod.

  - Radar

## Threat Counter-Tactics

- Surface-to-air.

  - Surface-to-air missile (SAM).

  - Air defense artillery.

- Air-to-air.

## Air tasking Order/Schedule Overview

- Supporting Air Assets.

  - Event/mission number.

  - Call sign.

  - Load out.

  - Time on station.

  - Capabilities and limitations.

- Surface-to-surface Assets.

  - Unit.

  - Location.

  - Communication plan.

  - Capabilities.

- Command, Control, and Communications Specifics.

  - Requirements.

  - Controlling agencies.

  - SCAR common.

  - Primary and alternate frequencies.

## Working Area/Map/Imagery Study

- SCAR working area/named area of interest (NAI)/target area of interest (TAI).

- Airspace coordinating measure (ACM).

## Ingress Routing

- Route.

- Formation/altitude/airspeed.

- Aircraft lighting.

- Timing.

- Fuel.

- Sensor/navigation update plan.

- Command, control, and communication requirements.

- Threat updates.

- Coordination.

- Deconfliction.

- Reactions to threats.

- Emissions control.

## SCAR Plan

- Overview.

  - Assigned SCAR working area.

  - Geographic orientation.

  - Target priorities.

  - (4) Expected target locations.

  - (5) Applicable commander’s critical information requirements (CCIRs).

- Reconnaissance profile.

  - System Preparation.

    - Waypoints.

    - Sensors.

  - Sanitation pass.

  - Search.

    - Formation/altitude/airspeed.

    - Aircraft lighting.

    - Lead/wingman contracts.

    - Visual/NVDs/binoculars.

    - Sensor plan.

    - Sensor predictions.

- Targeting.

- Rules of engagement (ROE), positive identification (PID), and collateral damage estimate (CDE).

- Passing/entering coordinates.

- Sorting.

- Talk-on/marks (laser codes).

## SCAR Coordination

- SCAR Asset Check-in.

  - Call sign.

  - Mission number.

  - Number/type.

  - Position.

  - Ordnance (laser code).

  - Time on station (TOS).

  - Type sensors.

  - Abort.

- Situation Update Brief.

  - Threat.

  - Enemy situation.

  - Friendly situation.

  - Artillery.

  - Coordination.

  - Hazards.

  - Remarks/restrictions.

- Reconnaissance.

  - Geographic Area Allocation.

    - Grid/Terrain Feature.

    - Vertical.

    - Timing.

    - Deconfliction.

  - Priority Targets.

    - Priority.

    - Expected target locations.

  - Applicable CCIRs.

- Target Engagement.

  - Holding.

    - Location.

    - Orientation.

    - Altitude.

  - Deconfliction.

    - Lateral/geographic.

    - Altitude.

    - Timing.

  - Targeting plan.

  - Marks/laser codes.

## SCAR Tasking

- Target attack brief.

- Attack geometry.

- Attack/Weapons Selection.

  - Planned weapon-to-target pairing.

  - Reactive weapon-to-target pairing.

- Weapons check.

- Formation/altitude/airspeed.

- Sensor plan.

  - Visual/NVDs.

  - Laser spot tracker.

  - Electro-optical/Infrared (EO/IR) sensor.

  - Radar.

- Delivery mode (primary/alternate).

- Moving target considerations.

  - Expendable use.

- Communication cadence.

- Reaction to threats.

- Abort criteria.

  - ROE, PID, CDE.

  - CAPTURE/SPOT.

  - Expendables BINGO.

- Off-target.

  - Geometry and flow.

  - Expendable use.

  - Communication cadence.

  - Deconfliction.

  - Shooter-cover role-swap.

  - Sensor plan.

  - Visual/NVDs.

- Reattack criteria/plan.

- Battle damage assessment (BDA).

## Battlefield Handover

- Threat.

- Enemy situation.

- Friendly situation.

- Artillery.

- Coordination.

- Hazards.

- Remarks/restrictions

## Egress

- Flow/rejoin.

- Route/deconfliction.

- Formation/altitude/airspeed

- Aircraft lighting.

- Timing.

- Fuel.

- Command, control, and communication requirements.

- Inflight report.

- Coordination.

- Reaction to threats.

- Emissions control.

## Contingencies

- DEADEYE.

- Asset fallout.

- Separated wingman.

- Weather.

- Battle damage.

- Hung ordnance.

- Personnel recovery.

- Contested, degraded, operationally limited operations

# SCAR DECONFLICTION CARD

________________________________________________________________________ ________________________________________________________________________ ________________________________________________________________________ ________________________________________________________________________ _____________________________________________________________________

CALLSIGN/MDS CHK IN: OFF STA:

CALLSIGN/MDS CHK IN: OFF STA:

CALLSIGN/MDS CHK IN: OFF STA:

CALLSIGN/MDS CHK IN: OFF STA:

CALLSIGN/MDS CHK IN: OFF STA:

CALLSIGN/MDS CHK IN: OFF STA:

# SCAR ATTACK BRIEF (DLER Format)

| MKPT | DESCRIPTION | LOCATION | ELEV | REMARKS/ RESTRICTIONS | PRIORITY | ASSIGNED AREA |
| --- | --- | --- | --- | --- | --- | --- |