DCS: Mi-24P Hind | Landing

Shallow Descent

Start a shallow decent from 3,000 meters or less at 140 to 150 kph, and a VVI of 5 to 7 meters per second. Rotor RPM should be 95% -/+ 2%.

Running Landing

Used when there is not sufficient engine power for a vertical decent, and there is available room for a running landing.

Descend between 130 and 70 kph, with 80 kph being the ideal approach speed.

At 50 meters and less than 150 kph, lower the landing gear and start pitch up to slow down with a VVI of about 3 meters per second.

At 40 to 50 meters, maintain a VVI of around 3 meters per second.

At 3 to 4 meters, airspeed should be further reduced to 50 kph, and not exceed 80 kph, with a VVI of 0.5 meters per second.

Smoothly land first on the main wheels, reduce collective pitch to 6- to 8-degrees.

Apply wheel brakes smoothly. To not input aft cyclic as it may result in a tail strike.

Once stopped, neutralized the cyclic and collective to minimum.

Vertical Descent

Use when the landing area is limited or obstacles around it.

At an altitude between 200 down to 20 meters, do not exceed a VVI of more than 3 meters per second.

At 50 meters, lower the landing gear.

Between 20 and 10 meters, slow descent to 3 meters per second. Below 10 meters, slow to 2 meters per second and at touch down, .2 meters per second.